No.9

F5| Application Connector

BD Cloud Apps

Table of Contents

Application Connector Configuration Part 1	3

Navigate to Github | https://github.com/gregcoward/awstraining/	3

Login to Your Aws Console | https://federate.f5.com/my.policy	3

Service Center Configuration	5

Connect to Your Assigned BIG-IP Via SSH	5

Login to the GUI and provision iRulesLX	6

Upload the App Connector RPM	7

Navigate To Github |  https://github.com/gregcoward/awstraining	7

Configure the Irulelx And Workspace for the Service Center Traffic	8

Create a Virtual Server and Dummy Pool	9

Configure the iAppLX Application to Always Be Running.	10

Authorize the App Connector	11

Authorize Application Connector	12

Add Application Nodes to Service Center	13

# Application Connector Configuration Part 1

## Navigate to Github | https://github.com/gregcoward/awstraining/

- Find the template that matches this lab # | Click to Open

- Find the menu bar | Click the button marked Raw

- Select all | Then Copy

- Paste contents into a text editor | Save as lab9.json

## Login to Your Aws Console | https://federate.f5.com/my.policy

- From the webtop | Select AWS Management Console

- Under the Management Tools menu| select CloudFormation

- Click the button  |Select the Radio button labeled Upload a template to Amazon S3 | browse to lab9.cft.json | Click

|  | Stack Name ¦ *user- lab # |  |
| --- | --- | --- |
|  | appConnectorName ¦ ac-*user |  |
|  | appInstance1Name ¦ this value is hardcoded |  |
|  | appInstance2Name ¦ this value is hardcoded |  |
|  | KeyName ¦ Select the key that you created in Lab 5 from the dropdown |  |
|  | adminUsername ¦ *user |  |
|  | adminPassword ¦ F5labnet |  |
|  | appConnectorType ¦ Select an option from the dropdown |  |
|  | appConnectorAMI ¦ Select an option from the dropdown |  |

- Fill in the parameters on the Specify Details page| Click

- On the Options page | Click

- On the Review page | Click

- You will be returned to the Cloud formation page where you can monitor the Status of your deployment

- If you click the Stack Name | Stack Details will display | Periodically refresh your bowser

- The Stack should complete in 10-15 minutes | you will see the  message in the Status column

# Service Center Configuration

## Connect to Your Assigned BIG-IP Via SSH

- You will have been assigned

  - One *BIG-IP

  - Three public IP address’s

- SSH to the BIG-IP CLI

  - Login: root

  - Password: F5labnet

- Type the following command | touch /var/config/rest/iapps/enable

## Login to the GUI and provision iRulesLX

- Use the first public IP address that you were assigned to access the GUI of your BIG-IP

- Navigate to | System | Resource Provisioning

- Nominally Provision iRules Language Extensions (iRulesLX)

## Upload the App Connector RPM

### Navigate to Github |  https://github.com/gregcoward/awstraining

- Find and click | app-connector-0.0.1-53.noarch.rpm

- Click |  and save on your local system

- Navigate back to your BIG-IP | Click iApps | Package Management LX | click Import | Browse to the Location where the RPM was saved in the previous step |Click Upload

It is anticipated that this package will be downloadable from downloads.f5.com or will come on all iSeries boxes pre-installed

| Note: You may receive the following error message |  |
| --- | --- |
| if so please click Package Management LX ensure that it installed successful |  |

## Configure the iRulesLX And Workspace for the Service Center Traffic

- Click on Local Traffic | iRules | LX Workspaces

- Verify that the application_connector workspace already exists from the installation of the RPM package

- Click | LX Plugins tab

- Click | Create button and use the Exact name application_connector_plugin, use drop down for From Workspace choosing application_connector | finally Click the finished button

### Create a Virtual Server and Dummy Pool

- On the BIG-IP navigate to | Local Traffic | Virtual Servers | Click Create | Use the table below for guidance

|  | Name | ac_vip |  |  |
| --- | --- | --- | --- | --- |
|  | Provide a unique value for the VIP used to communicate with Service Center |  |  |  |
|  | Destination Address | 2nd Public address assigned to your lab |  |  |
|  | Type in a VIP address that the App Connector will be able to reach over the Internet |  |  |  |
|  | Service Port | 443 |  |  |
|  | Select a port, in the case of a single NIC BIG-IP make sure it is not the mgmt port or port 443, so that it does not interfere with regular VIP's |  |  |  |
|  | SSL Profile | clientssl-insecure-complatible |  |  |
|  | Add a Client SSL profile like clientssl-insecure-compatible "for now" |  |  |  |
|  | Default Pool | Click the Plus |  |  |
|  | The settings in the pool do not need to exist and the information below is an example of values you can use |  |  |  |
|  | Name | dummy_pool |  |  |
|  | Health Monitor | none |  |  |
|  | Node Name | dummy_node |  |  |
|  | address | 192.168.1.115 | Click |  |
|  | Service Port | 80 | HTTP | Click |
|  | Virtual Server List ¦ Click |  |  |  |

## Configure the iAppLX Application to Always Be Running.

- Click on iApps | Application Services | Applications LX | Click

- You only need to click the drop down and select the app-connector iApp.

  - The Service Center will load

  - You will be prompted for credentials

  - Use the same credentials you used to login to the BIG-IP to login to the Service Center.

- Click

  - User Name is pre-populated

  - Common partition is pre-populated

  - App Name field | Provide a value like AppConServCent

  - Click the green button marked Save

- Verify this by clicking on iApps | Application Services | Applications LX |Deploy the newly created service by selecting the checkbox | Then select

  - Note the Green ball  Service Center will remain running

- Navigate back to Application LX | and select the application previously created

  - Select Config

  - Select the Proxy Virtual Server tab

  - Check the check box next to the newly created virtual server

  - Select the Save button

  - Then select Yes to Confirm

# Authorize the App Connector

- Return to the AWS Console | Click CloudFormation | Select the checkbox next to the Stack Name that you had previously deployed | Click the tab near the bottom labeled Outputs | Click App Connector URL link to connect to the application connector.

- Use the username and password when you initially deployed the CFT to login to the Application Connector

Note: If you are using Microsoft Edge you may not see the credential prompt. Switch to another browser like Chrome

- In the Service Center Connections Section | Click on the  button

  - Name | lab#bigip

  - IP/FQDN | Provide the Virtual Server IP address

  - Port  | 443

  - Click the connect icon

- Click OK on the dialog box saying the App Connector was added Successfully

# Authorize Application Connector

- Login to the BIG-IP with the Service Center Running on it.

  - Click iApps | Application Services | Applications LX

  - Click your Service Center Application

  - Login with the BIG-IP credentials provided.

  - Click on the Proxies Tab

  - Select the check mark next to the Application Connector that you added earlier.

  - Click    | Successful authorization will turn the status ball from red to green.

# Add Application Nodes to Service Center

- You will need to return to your AWS Portal | log into the Application Connector | Use the username and password you entered in the CFT

  - In the Nodes section | click the Add button | You will need to enter the information that is separated by double hashes ## as shown in the next step

  - You can find the application servers information in the Outputs section of the CloudFormation Stack

  - Enter Description | Then click  button appears near the end of the row

  - Check the checkboxes next to the nodes | Click  to activate the nodes.

- login into the Service Center | Local Traffic | Nodes | Verify that the nodes are active by click on the nodes tab in the Service Center

- Both nodes have been added, and should be visible in Local Traffic | Nodes.

- Add the nodes to | a Pool | and Virtual Server

- Test your connection